Last modified: 12-20-96

Setting up your DI-300(M) to Talk to a Cisco Router

I.	Cisco Router dialing to DI-300(M) Router

Cisco

Router

DI-300(M) Router

W/S

W/S

W/S

W/S

- Setup Presitge/Cisco for PAP - If Cisco requests PAP in LCP configure-request in an incoming call to us, we must not provide a user/password that can be used to log in to our system.

- In menu 13, set the Mutual Authen field to ‘Yes’.

- Set the PAP Login to the username for this authentication.    This username must be different from any other username (remote node or remote user) defined in the DI-300(M).

- In the PAP Password field, enter the password for that user.

- Setup DI-300(M)/Cisco for CHAP - If Cisco requests CHAP in the LCP configure-request in an incoming call to the DI-300(M), you need to define a “dialer map” command for the Cisco calling device.    For instance,

hostname csco2503
			username 306t password abc
			dialer map ip 204.247.203.151 name 306t 7770408

This command maps an IP address to a telephone number to dial.    The optional ‘name’ restricts the connection to be a particular host name.

For the DI-300(M), you need to make the following configurations:

- In menu 11, set the Rem Login field to the hostname of the Cisco device.

- Also in menu 11, set the My Login field to the same name used for the System Name field in menu 1.

- In menu 11, make sure that the Rem Password and My Password fields have the same password that the Cisco device is expecting.

II.	DI-300(M) dialing to Cisco Router

- Setup DI-300(M)/Cisco for CHAP - If the DI-300(M) is the calling device, then no special configuration paramters need to be modified as this is the same scenario as calling any other remote node.