Last modified: June 12, 1997

Telecommuting/Remote Access W/S with a Dynamically Assigned IP Address to Route IP

192.68.135.0/24

ISDN TA

DNS

DI-300(M)

W/S with

TCP/IP

192.68.135.1          192.68.135.2

[‘204.5.1.1’ - dynamic IP address]

Note: The IP addresses shown above are examples only!

This configuration note explains how to set up a workstation using an ISDN terminal adapter (TA) to connect to the DI-300(M) to route IP.    In this configuration, the workstation has TCP/IP software installed.    Once connected, the workstation will able to perform any TCP/IP applications on the network stations (e.g., FTP, telnet, etc.).    There will be two items that you need to set up.    These are the workstation and the DI-300(M).

I.	Workstation Setup

To set up the workstation (W/S), you will need to set the following parameters:

- IP Address - the IP address in this case will be dynamically assigned by the DI-300(M).    Generally, you should simply enter ‘0.0.0.0’ into the IP address field.

- DNS (Domain Name Server) Address - the IP address of the server station on the remote network that acts as the DNS.    In the above example, ‘192.68.135.2’.

- Default Gateway - the IP address of the station or device on the remote network that acts as your default gateway.    That is, any packets without an implicit route to their destination IP address will be routed to the default gateway.    In our example, the Default Gateway would be the router you are connected to, ‘192.68.135.1’.

The procedure for configuring these parameters for your workstation may differ depending on the type of TCP/IP networking software you are using on your workstation.    If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- Trumpet WAN Setup - ISDN TA to DI-300(M) 2864I

- Chameleon WAN Setup - ISDN TA to DI-300(M) 2864I

- Windows 95/NT TCP/IP WAN

- MacTCP Setup

II.	DI-300(M) Setup

In order to configure your DI-300(M) for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= No

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IP field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= None

Client IP Pool Starting Address= N/A

Size of Client IP Pool= N/A

Primary DNS Server= N/A

Secondary DNS Server= N/A

TCP/IP Setup:

IP Address= 192.68.135.1

IP Subnet Mask= 255.255.255.0

RIP Direction= Both

Version= RIP-v1

Press ENTER to Confirm or ESC to Cancel:

In menu 3.2,    the IP Address and IP Subnet Mask field must be set correctly.

Menu 13:

Menu 13 - Default Dial-in Setup

Telco Options:                       IP Address Supplied By:

CLID Authen= None                    Dial-in User= No

IP Pool= Yes

PPP Options:                             IP Start Addr= 204.5.1.1

Recv Authen= CHAP/PAP                  IP Count(1,2)= 1

Compression= No

Mutual Authen= No                  IPX Net Num Supplied By:

PAP Login= N/A                     IPX Pool= No

PAP Password= N/A                    IPX Start Net Num= N/A

Multiple Link Options:                 IPX Count(2,16)= N/A

Max Trans Rate(Kbps)= 128

Session Options:

Callback Budget Management:            Input Filter Sets=

Allocated Budget(min)= 0             Output Filter Sets=

Period(hr)= 0                        Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

- The Recv Authen field should be set to the type of authentication protocol you want to use.

- Since the workstation needs to have its IP address assigned, set the IP Address Supplied By: Dial-in User field to ‘No’.

- Make sure that IP Pool is set to ‘Yes’.

- In IP Start Addr, enter the IP address that you want to assign to the workstation when it dials in.    In our example, this would be ‘204.5.1.1’.

- Set IP Count(1,2) to ‘1’.

Menu 14:

Menu 14.1 - Edit Dial-in User

User #: 1

User Name= username

Active= Yes

Passwd= *******

Callback= No

Phone # Supplied by Caller= N/A

Callback Phone #= N/A

Rem CLID=

Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

- The User Name and Passwd fields should be set to the login user name and password that the workstation will provide when dialing in to the DI-300(M).

- Set the Active field to ‘Yes’.