Last modified: June 13, 1997

DI-300(M) dialing to Windows NT using Remote Access Administrator w/ ISDN Terminal Adapter

This document describes how to setup the DI-300(M) and the Windows NT workstation for the following application:

DI-300(M)

ISDN

TA

ISDN

Win NT

Setting up a workstation on your network to use the DI-300(M) to connect to a workstation (e.g., Windows NT 4.0 client using Remote Access Administrator) with an ISDN Terminal Adapter to route IP and/or IPX.

I.	Setup Windows NT (Must have administrator level access)

A.	Enable TCP/IP and/or IPX on the WAN sideSelect Network Control Panel

1.	Add the Remote Access Service to the Network:Service menu

- Configure Remote Access.    Configure the TCP/IP and IPX protocols as desired.    Make sure to select the option ‘Allow any authentication including clear text’

3.	Save your changes and reboot computer when prompted

B.	Go to Startup:Remote Acess:Remote Access Admin

1.	Select Servers: Start Remote Access Server and configure

2.	Select Users:Privileges

3.	Give needed users dial in access

- If callback wanted, select your callback option* (If using “Preset To” being sure to include any prefixes needed (eg. by a PBX))

*Because the DI-300(M) usually will be stationary it is highly recommended to use the ‘Preset To’ as this increases the security of the network

II.	DI-300(M) Setup

In order to configure your DI-300(M) correctly for this application make sure you have the following

menus configured correctly.

Note: Fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= DI300

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= No

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IP field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).    The 1st Phone # field will be the first number for NT to call back.    The 2nd Phone # field will be the second number for NT to callback.    The second number will be used to send to NT if you specify the Callback option ‘Set by Caller’.    Be sure to include any prefixes and area codes that it may need to callback.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= None

Client IP Pool Starting Address= N/A

Size of Client IP Pool= N/A

Primary DNS Server= N/A

Secondary DNS Server= N/A

TCP/IP Setup:

IP Address= 204.247.203.175

IP Subnet Mask= 255.255.255.192

RIP Direction= In Only

Version= RIP-v1

Press ENTER to Confirm or ESC to Cancel:

- In menu 3.2,    the IP Address and IP Subnet Mask field must be set correctly in order for your workstation to communicate with your DI-300(M).

Menu 11:

Menu 11.1 - Remote Node Profile

Rem Node Name= To_NT	               Route= IP

Active= Yes                          Bridge= No

Call Direction= Both

Edit PPP Options= No

Incoming:                            Rem IP Addr= 204.5.1.1

Rem Login= NT                      Edit IP/IPX/Bridge= No

Rem Password= *******              Telco Option:

Rem CLID=                            Transfer Type= 64K

Call Back= No                        Allocated Budget(min)= 0

Outgoing:                              Period(hr)= 0

My Login= username                 Session Options:

My Password= ********                Input Filter Sets=

Authen= CHAP/PAP                     Output Filter Sets=

Pri Phone #= 5551214                 Call Filter Sets=

Sec Phone #=                         Idle Timeout(sec)= 300

Press ENTER to Confirm or ESC to Cancel:

- For the remote node name and set the Active field to ‘Yes’.

- The Call Direction field should be set to ‘Both’.

- My Login and My Password is the NT account name and password to login to NT.

- Pri & Sec Phone # is the phone number of the ISDN TA you are dialing to.

- The Route field should be set to the protocols you want to route.

- The Rem IP Address is the workstations IP address. *NT 4.0 beta 1 claimed the first address in it’s ip pool, set under Network: Services: Remote Access as it’s own address (this is an NT bug).

Menu 13:

Menu 13 - Default Dial-in Setup

Telco Options:                       IP Address Supplied By:

CLID Authen= None                    Dial-in User= Yes

IP Pool= No

PPP Options:                             IP Start Addr= N/A

Recv Authen= None                      IP Count(1,2)= N/A

Compression= Yes

Mutual Authen= No                  IPX Net Num Supplied By:

PAP Login= N/A                     IPX Pool= No

PAP Password= N/A                    IPX Start Net Num= N/A

Multiple Link Options:                 IPX Count(2,16)= N/A

Max Trans Rate(Kbps)= 128

Session Options:

Callback Budget Management:            Input Filter Sets=

Allocated Budget(min)= 0             Output Filter Sets=

Period(hr)= 0                        Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

- The Recv Authen field must be set to ‘None’ if you configure NT to call back to DI-300(M).