Last modified: June 10, 1997

Internet Access for Workstation/PC where ISP assigns a dynamic single IP address

ISP Gateway

192.68.135.1

204.5.1.1

W/S

192.68.135.3

DI-300(M)

DNS

[dynamic address]

192.68.135.2

Note: The IP address shown above are examples only!

This configuration note explains how to set up a workstation on your network to use the DI-300(M) to connect to the Internet via an ISP.    In this configuration, the user has a single user Internet account which will assign one IP address dynamically.    Notice that with this configuration, all stations on the network will be able to access the Internet, but they will be ‘hidden’ from the outside.    That is, from the ISP’s point of view, they will only be able to ‘see’ the single IP address (in this case, the one dynamically assigned)    There will be two items that you need to set up.    These are the workstation and the DI-300(M).

I.	Workstation Setup

To set up the workstation (W/S), you will need to set the following parameters:

- IP Address - the IP address assigned to the workstation itself, in the above example, ‘192.68.135.1’

- Subnet Mask - the subnet mask used for your network.    Class C networks generally use a 24-bit netmask, ‘255.255.255.0’.

- DNS (Domain Name Server) Address - the IP address of the server station on your network that acts as the DNS.    In the above example, ‘192.68.135.2’.    Please note that some networks will not have a DNS.    In this situation, you will need to obtain the IP address of a remote DNS from the ISP.

- Default Gateway - the IP address of the station or device on your network that acts as a default gateway.    That is, any packets without an implicit route to their destination IP address will be routed to the default gateway.    Since the DI-300(M) is connecting to the Internet, the default gateway in this case would be the IP address of the DI-300(M), ‘192.68.135.3’.

The procedure for configuring these parameters for your workstation may differ depending on the type of TCP/IP networking software you are using on your workstation.    If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- Trumpet Winsock Windows 3.1 Workstation Setup - see section ‘Internet Access’

- Chameleon LAN Setup - see section ‘Internet Access’

- Windows 95/NT - see section ‘Internet Access’

- MacTCP Setup

II.	DI-300(M) Setup

In order to configure your DI-300(M) for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= P2864I

Location= Taipei

Contact Person's Name= CC

Route IP= Yes

Route IPX= N/A

Route AppleTalk= N/A

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IP field is set to ‘Yes’.

Menu 2:

Menu 2 - ISDN Setup

Switch Type= Northern Telecom Custom

B Channel Usage= Switch/Switch

1st Phone #= 5551212

SPID #= 408555121200

2nd Phone #= 5551213

SPID #= 408555121300

Press ENTER to Confirm or ESC to Cancel:

- In menu 2, set your switch type, phone numbers, and SPIDs (if necessary).

- After saving this menu, you will be asked if you want to perform an ISDN connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= Server

Client IP Pool Starting Address= 192.168.0.1

Size of Client IP Pool= 20

Primary DNS Server= 38.1.1.10

Secondary DNS Server= 0.0.0.0

TCP/IP Setup:

IP Address= 192.68.135.3

IP Subnet Mask= 255.255.255.0

RIP Direction= Both

Version= RIP-v1

Press ENTER to Confirm or ESC to Cancel:

- In menu 3.2,    the IP Address and IP Subnet Mask field must be set correctly in order for your workstation to communicate with your DI-300(M).

Menu 4:

Menu 4 - Internet Access Setup

ISP's Name= My_ISP

ISP IP Addr= 204.4.5.1

Pri Phone #= 5551212

Sec Phone #=

My Login= username

My Password= ********

Single User Account= Yes

IP Addr= 0.0.0.0

Server IP Addr= 192.68.135.2

Telco Option:

Transfer Type= 64K

Press ENTER to Confirm or ESC to Cancel:

- The ISP IP Addr field should be set to the ISP Gateway address.    This address is supplied by your ISP.

- Pri Phone # is the number your DI-300(M) has to dial in order to access your ISP.    This number is supplied by your ISP.

- My Login and My Password is the login information provided by your ISP.

·	Since you have a single user Internet account, Single User Account should be set to ‘Yes’.

- For the IP Addr field, since the IP address will be dynamically assigned, enter ‘0.0.0.0’ or you can leave this field blank.

- Server IP Addr should have the IP address of the server station on your network.    Please note that this configuration will only work if you have one server on the network.    In our example, the DNS is our server, so we would enter ‘204.5.1.2’.    If you do not have a server station on your network, just leave this field blank.

- After saving this menu, you will be asked if you want to perform an Internet connection test.    Select ‘Yes’ to perform the test.    If the test fails, refer to the User’s Manual Troubleshooting section for corrective action.

Menu 11:

Menu 11 - Remote Node Setup

1. My_ISP (ISP)

2. ________

3. ________

4. ________

Enter Node # to Edit:

After you have configured and saved menu 4, you should see that you have created a remote node in menu 11.    You can perform more advanced configuration options to this remote node in this menu.    In addition, you can make manual calls to the Internet by using the Manual Call option in menu 24.4 and selecting the corresponding ‘(ISP)’ remote node.