Procedure for Internet Explorer 8

----------------------------------

The maximum number of connection per server should be limited to 1 The only problem is that MSIE 8 stores that setting on a different location compared to MSIE 7 and older versions.

According to this MS article:

http://support.microsoft.com/kb/282402

You have to follow procedure with version 8:

=============================================================

1. Start Registry Editor.

2. Locate the following key in the registry:

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\InternetExplorer\MAIN\FeatureControl\FEATURE_MAXCONNECTIONSPERSERVER

3. On the Edit menu, point to New, click DWORD Value, and then add the following registry values:

Value name: iexplore.exe

Value data: 10

Base: Decimal

Note set this value to the connection limit that you want for HTTP 1.1 connections. By setting the value to 10, you increase the connection limit to 10.

4. Locate and then click the following registry subkey:

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\InternetExplorer\MAIN\FeatureControl\FEATURE_MAXCONNECTIONSPER1_0SERVER

5. On the Edit menu, point to New, click DWORD Value, and then add the following registry values:

Value name: iexplore.exe

Value data: 10

Base: Decimal

Note set this value to the connection limit that you want for HTTP 1.1 connections. By setting the value to 10, you increase the connection limit to 10.

6. Exit Registry Editor.

=============================================================

It solves the web interface problem on this switch, obviously set the connections to 1 and not to 10.