DOCUMENTATION DATABASE

DATE:11/21/97

CODE       : 3993927Q

PROGRES.   : 501

PRODUCT    : PC

MODEL      : M4-40,M4-46

TYPE       : FCO

DATE BEGIN : 11/01/93

PAGES      : 3

TITLE      : HW EVOLTION FOR TROUTE BOARD FROM LEV 04

TO LEV 05 TO LEV 07

NEW BIOS 2.0 TO CORRECT A PROBLEM DETECTED WITH OS/2, REL. 1.3 VER. 2.02 UPD.6 PRINTED CIRCUIT EVOLUTION

ORGANIZATION

A new  printed circuit  level  01, code  654192S, will  be  installed  at factory on trout 1 boards BA2034,  BA2032, BA2039, starting from the  end of October 1993. Following this  modification, the level of the  M4-4X/6X PC boards will change as follows:

BA2032:     from 04 to 05

BA2034:     from 05 to 06

BA2039:     from 05 to 06

A new BIOS rel. 2.0 will be installed  on the same boards, starting  from November 1993.

The installation of the new BIOS calls for the following modifications to be carried out:

- add three tracks

- make two cut outs

- remove one resistance

- install one resistance

- install one mini vip, 3-pin jumper.

In view of the above, the HW level of the boards will change as follows:

BA2032:  from 05 to 06

BA2034:  from 06 to 07

BA2039:  from 06 to 07

The following parts are available at DRS:

BIOS 2.0, code 588288 H

3-pin jumper, code 5785920 S

4.7 Kohm resistance, code 4901605 B

DESCRIPTION

On the new printed circuit  level 01, the eight  preexisting tracks  have been absorbed to correct the parity  error problem described in FCO  500, and appropriate bays have been provided for capacitors Ca1, Ca2, Ca3 and Ca4, which were previously held into position by a double-stick tape.

By installing  the  new  BIOS  and  carrying  out  the  HW  modifications described above, the defect previously  detected with the  OS/2 rel. 1.3 operating system,  which  was  causing  an  unclean  screen form to be displayed when loading the first disk, will no longer occur.

This was  caused  by  the  fact that  the  on-board  VGA  controller  was addressed to port 3C3h while a second controller installed on the bus was addressed to port  46E8h. With the new BIOS, both controllers  will be addressed to port 46E8h.

EXECUTION

- Remove the 6.8 Kohm resistance in position R156.

- Install a 4.7 Kohm resistance in position R270.

- Install a mini vip, 3-pin jumper in position J45.

- Install the EPROM with the new BIOS 2.0.

With reference to the attached diagram:

On side A:

- Make one cut out on the track of pin 41 of U41 (signal name: 	reset4).

- Add one track between pin 1 of J45 and reference B (signal name:  	reset 4).

- Add one track between pin 2 of J45 and pin 41 of U41.

On side B:

- Make one cut  out on  track (GNO)  starting  from pin  2 of  R270  	(see reference C).

- Make one cut out on the track starting from  pin 2 of J45, signal  	name MVD(16) (see reference D).

NOTE: Pins 1-2 on  jumper J45  are  deactivated by  default. If  a  video controller is installed on  the bus,  pins 2-3 on  jumper J45  will also have to be deactivated.