DOCUMENTATION DATABASE                             DATE:11/21/97

CODE       : 3993927Q

PROGRES.   : 501/A

PRODUCT    : PC

MODEL      : M4-40,M4-46,M4-62,M4-64,M4-66

TYPE       : FCO

DATE BEGIN : 10/30/93

PAGES      : 4

TITLE      : HW EVOLUTION OF TROUT 1.5 BOARD

NEW BIOS 2.0 TO CORRECT A PROBLEM DETECTED WITH OS/2, REL 1.3

VER. 2.02 UPD. 6

PRINTED CIRCUIT EVOLUTION

ORGANIZATION

A new printed  circuit level  01,  code 654192S,  has been  installed  at factory, in October 1993, on trout 1 boards BA2034, BA2032, BA2039.

Following this modification,  the level  of the M4-4X/6X  PC boards  will change as follows:

BA2034:     from 06 to 07

BA2039:     from 06 to 07

A new BIOS  rel. 2.0  has been installed  since November  1993 on  boards BA2034 and BA2039.

The installation of the new BIOS calls for the following modifications to be carried out:

- add three tracks

- make two cut outs

- remove one resistance

- install one resistance

- install one 3-pin, mini vip jumper.

In view of the above, the HW level of the boards will change as follows:

BA2034:  from 05 to 06

BA2039:  from 05 to 06

The following parts are available at DRS:

BIOS 2.0, code 588288 H

3-pin jumper, code 5785920 S

4.7 Kohm resistor, code 4901605 B

Board  BA2032  wil  be  subject  to  the  same  modifications,  with  the difference that, first of all, the new BIOS 2.0 release will be installed to correct the OS/2 problem, and this will change the board's level from 04 to 05; afterwards, the new printed circuit will be installed, and this will change the board's level from 05 to 06.

DESCRIPTION

On the  new printed  circuit,  the eight  preexisting  tracks  have  been absorbed to correct the parity  error problem described in  FCO 500, and appropriate bays have been provided for capacitors Ca1, Ca2, Ca3 and Ca4, which were previously held into position by a double-stick tape.

By installing  the  new  BIOS  and  carrying  out  the  HW  modifications described above, the defect previously  detected with the  OS/2 rel.  1.3 operating system,  which  was  causing  an  unclean  screen  form to be displayed when loading the first disk, will no longer occur.

This was  caused  by  the  fact that  the  on-board  VGA  controller  was addressed to port 3C3h while a second controller installed on the bus was addressed to port  46E8h. With  the new BIOS,  both controllers  will be addressed to port 46E8h.

EXECUTION

- Remove the 6.8 Kohm resistor in position R156.

- Install a 4.7 Kohm resistor in position R270.

- Install a 3-pin, mini vip jumper in position J45.

- Install the EPROM with the new BIOS 2.0.

With reference to the attached diagram No.1 relative to the cut outs  and tracks:

On side A:

- Make one cut out on the track of pin 41 of U41 (signal name: reset4).

- Add one track between pin 1 of J45 and reference B (signal name:  reset

4).

- Add one track between pin 2 of J45 and pin 41 of U41.

On side B:

- Make one cut  out on  track (GND)  starting  from pin  2 of  R270  (see reference C).

- Make one cut out on the track starting from  pin 2 of J45, signal  name MVD(16) (see reference D).

NOTE: Pins 1-2 on  jumper J45  are  deactivated by  default. If a  video controller is installed on  the bus,  pins 2-3 on  jumper J45  will also have to be deactivated.