DOCUMENTATION DATABASE

DATE:11/21/97

CODE       : 3993927Q

PROGRES.   : 505

PRODUCT    : PC

MODEL      : M4-40,M4-46,M4-60,M4-62,M4-64,M4-65,M4-66

TYPE       : FCO

DATE BEGIN : 12/30/94

PAGES      :

TITLE      : MOTHER BOARD EVOLUTION

BOARD TROUT 1 INSTALLED ON THE SYSTEM SHOWS VIDEO INSTABILITY WHEN WORKING IN THE OS/2 AND WINDOWS ENVIRONMENTS ENHANCED BY THE PRESENCE OF A OVERDRIVE

ORGANIZATION

In November 1994, a modification has been  brought to the TROUT 1  board, thus causing the following change in hardware levels:

Boards BA2032 and BA2039 will switch to level 16, while Board BA2034 will switch to level 15.

For all those boards that  did not  include the previous  OS/2, rev.  1.3 modification, the level will switch to 14/A and 13/A, respectively.

This modification mainly concerns TROUT 1 boards that host an  OVERDRIVE and that have been  installed on systems equipped with Micro Tower box (M4-40, M4-46).

The modification consists of replacing  the 1 Ohm  resistors in  position R148 and R149 with 0 Ohm resistors, DRS code 4901500F.

In addition to the above modifcation, it will also be necessary to  check the +5V on the power supply.

DESCRIPTION

In the OS/2 and WINDOWS  environments, it  is possible to  detect a  high degree of instability in the video background when accessing  an FDU  or HDU. This problems mainly concerns boards  enhanced by the additon of an OVERDRIVE and inserted in a  Micro Tower box. This is  even more so  when there are boards installed on the bus or if there is a CD-ROM present in the system configuration.

The cause is a drop  of the video controller'a analog  power supply  (DAC palette power supply) from 4.9V  to 4.6V whenever the  above modules are added to the system configuration.

The minimum threshold set by the specifications is 4.75V.

By changing the resistors' value,  the video controller's analog  voltage will increase of about 100 mV.

EXECUTION

On the board's side B (soldered side), replace the 1 Ohm resitors in position R148 and R149 with 0 Ohm resistors.

Disconnect the power cables from the peripherals.

As the +5.1V +/-10mV must be present on the power supply's connector, the load must be as accurate as possible.

On the +5 -3A, load a 1.6 Ohm +/-2%, 30 Watts resistor.

On the +12 -0.2A, load a 62 Ohm +/-2%, 5 Watts resistor.

Connect the resistors to the  power supply's connector  that connects  to the bus expansion board.

Measure the actual value on the +5V pin.