DOCUMENTATION DATABASE                             DATE:11/20/97

CODE       : 3993927Q

PROGRES.   : 503

PRODUCT    : PC

MODEL      : M4-40,M4-46,M4-60,M4-62,M4-64,M4-65,M4-66

TYPE       : FCO

DATE BEGIN : 04/01/94

PAGES      : 17

TITLE      : NEW BIOS 2.0 ON TROUT 1 BOARD IN SOLVING 					 PROBLEM WITH OS/2.

NEW BIOS VERSION 2.0 TO SOLVE A PROBLEM WITH OPERATING

SYSTEM OS/2 REL. 1.3 VER. 2.02 UPD. 6

EVOLUTION OF THE PRINTED CIRCUIT.

ORGANIZATION

Printed  circuit  board  code  654192S,  board  revision  level  01,  was introduced in production in October on trota1 boards: BA2034; BA2032; and BA2039.

As a result of this  the boards of  PCs M4-4X/6X evolve  to the  revision levels shown below:

BA2034 from level 06 to level 07

BA2039 from level 06 to level 07

Beginning November, trota1 boards BA2034; and BA2039, have been  produced   with BIOS release 2.0.

The installation  of  this  BIOS  release  on  the  boards  requires  the following modifications:

- 3 wirings

- 3 PCB track cuts

- the elimination of one resistor

- the installation of one resistor

- the installation of a "mini vip" three-way jumper.

These changes result in the  HW revision level changes of  the boards  as shown below:

BA2034 from level 05 to level 06

BA2039 from level 05 to level 06

The following parts are available from DRS:

BIOS version 2.0 quoting code 588288H

The three-way jumper quoting code 5785920 S

The 4.7 Kohm resistor quoting code 4901605 B.

The BA2032 board undergoes  the same modifications  described above  with the difference that the modification for OS/2 is introduced first and the board revision level changes  from  level  04 to  level  05,  while the adoption of the  new PCB  implies BA2032  board evolution  from  revision level 05 to level 06.

DESCRIPTION

The new printed circuit board eliminates the  8 wirings present due to a parity error problem described in FCO 500, component positions were  also created for capacitors: Ca1, Ca2,  Ca3, and Ca4; previously fitted  using double-sided tape.

The new BIOS, along with  HW modifications, allows operating system  OS/2 rel.1.3, to be installed correctly without the problems seen  previously,  (display problems during loading of the first disk).

This was due to the fact that previously the on-board VGA controller  was assigned address port 3C3h,  and any other video controller installed  on the BUS was  addressed at  port 46E8h. With  this BIOS  version, both controllers are addressed at 46E8h.

IMPLEMENTATION

- Eliminate the 6.8 Kohm resistor in position R156.

- In position R270 add a 4.7 Kohm resistor.

- "      "    J45 mount the three-way "mini vip" jumper.

- Install the EPROM with BIOS rel. 2.0.

With reference to attachment 1: breaks and wirings diagram

On side A perform:

- a cut on the track of pin 41 of U41 (signal name reset4)

- a wiring from  pin 1 of J45 and reference point B (signal reset4)

- "   "     "    pin 2 of J45 and pin 41 of U41.

On side B perform:

- a cut on the PCB track (GND) that leaves  pin 2 of component R270  (see  reference C)

- a cut  on the  track  from pin  2  of J45,  signal  name  MVD(16)  (see  reference D)

- a wiring between pin 2 of R270 and pin 1 of R177

NOTE: The default setting of jumper J45 is pins 1-2 cloed (on-board video  controller), close pins 2-3 if a video controller installed on  the bus is to be used.