DOCUMENTATION DATABASE

DATE:11/21/97

CODE       : 3993927Q

PROGRES.   : 504

PRODUCT    : PC

MODEL      : M4-40,M4-60

TYPE       : FCO

DATE BEGIN : 06/30/94

PAGES      : 2

TITLE      : NEW PCS,ELIMINATION OF NOISE ON IRQ14.FOR M4-4X/6X PCS.

REWORK OF THE MOTHERBOARD PRINTED CIRCUIT BOARD

ELIMINATION OF A DISTURBANCE ON SIGNAL IRQ14

ORGANIZATION

From June 94,  the BA2076/2077/2078/2079  boards pass  from level  07  to level 08 due to the  introduction of  a 1000pF SMD  case 1206  capacitor, code 5070073E, in location CA1.

The same modification can be applied to  the BA2032 trota 1 boards  which change to level 14 (level 12/A for  the boards that have not  implemented the modification for OS/2 rel. 1.3);  BA2034/2039 which pass to level 15 (level 13/A for boards which  have not implemented  the modification for OS/2 rel  1.3), and  trota  1.5 first  series  BA2050/2051/2063/2064/2068 which assume level 06.

On the BA2076/2077/2078/2079 boards,  a new printed  circuit board,  code 654265J/00, has been introduced which  eliminates all the trace cuts and wirings previously  implemented for  the  following problems:  WAN  (LPU) board; WANG, COGENT, COFAX boards; CPU Intel SL-E; Battery problem.

The introduction of the PCB makes the board level evolve to 09.

DESCRIPTION

On   motherboards   BA2076/2077/2078/2079   previous    to   level    08; BA2050/2051/2063/2064/2068 previous to level 06; BA2032 previous to level 14, BA2034/2039 previous to level 15;  BA2032 previous to level 12/A and BA2034/2039 previous to level 13/A.

When 170 MB  and 340 MB  HDUs supplied  by QUANTUM are  installed on  the system, a disturbance that may occur  on signal IRQ14 can compromise  the proper functioning  of the  system. In fact the disturbance  may be

interpreted as an extra Interrupt causing malfunctions during operations on the HDU.

IMPLEMENTATION

On  BA2076/2077/2078/2079 motherboards  previous to level 08 and  on

BA2050/2051/2063/2064/2068 motherboards previous to level 06.

On side B (solder side)

Install a 1000pF SMD case 1206  capacitor, code 5070073E, between pin  11 of U1 (IRQ14) and pin 1 of capacitor C411 (GND).

On the following motherboards: BA2032  previous to level 14,  BA2034/2039 previous to  level 15; BA2032 previous to level 12/A and  BA2034/2039 previous to level 13/A.

On side B (solder side)

Install a 1000pF SMD case 1206  capacitor, code 5070073E, between pin  10 of U1 (GND) and pin 1 of R6 (IRQ14).