| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. Beata |  |  | A. Niosi |  |  |  |  |  |  |  | Sep-97 |  |  |  |  | 2800392R |  |  |  |  | 01 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| S5 PC modulo |  |  | M2 233MT/266MT |  |  |  |  |  |  |  | generica |  |  |  |  | BA2377 |  |  |  |  |  |  |  |
| titolo / subject Nuova piastra Matrox Millenium Nuova rel. di CD Drivers Modifica sul gruppo ventilatore Evoluzione mother board BA2377 Evoluzione BIOS –––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  | - altri / other ––––– |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time 0.50 |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level 03 |  |  |  |  | livello minimo di aggiorn. minimum revision level 03 |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autorizzati per riparazioni in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |

DESCRIZIONE

Matrox Milennium

La piastra GO2089 va in phase out e viene rimpiazzata dalla nuova Millenium “Shrink” GO2168

disponibile con cod. 212779J.

Nel sito Internet http://www.ocwi.com/ihdesk/modulo/m2/sw_tool.htm

si trovano i nuovi drivers per Millenium rel.2.02 consistenti in 9 Image Files

Driver Collection

E’ stato rilasciato il nuovo CD driver collection rel. 1.20 (cod.2693876E) inserito nel nuovo Starter Kit per Window NT 4.0

Gruppo Ventilazione

- Sul Gruppo Ventilazione (quello anteriore in prossimita’ del Pentium II), il diodo zener da 3.9 V, montato sul filo di massa, rende critico  il segnale di “FAN FAULT”; Per ovviare al problema, dal mese di Maggio, il diodo e’ stato spostato sul filo del +12V (rosso) ed il suo valore cambia da 3.9 V a 3.3 V Cod. 4825021J. Con Questa modifica si e’ migliorato anche il controllo della rotazione della ventola in funzione della temperatura ambiente.  Qualora non si reperisca il diodo con il valore corretto è comunque conveniente spostare quello  esistente sul filo di massa (nero) su quello del +12V (rosso).

-Ad esaurimento scorte in produzione la ventola di raffreddamento attualmente utilizzata, verra’ sostiuita con un’altra termoregolata.

I fornitori sono NMB e INNOvative ed il codice per richiederle a DRS e il 216678R

Evoluzione Mother Board BA2377

La moter board BA2377 (Dublin) è stata installata sui sistemi Modulo M2 a partire

dal livello Intel AA669523-503, (livello Olivetti NA); di seguito sono elencate le sue evoluzioni

e le relative modifiche.

1-Con l’introduzione di 2 resistenze atte a migliorare il segnale WOL proveniente dal connettore di

wake-up della LAN esterna, il codice intel passa da 503 a 504 - il codice Olivetti passa da NA a 01

2-L’aggiornamento della documentazione di mother board che integra alcune informazioni

gia’ riportate nel documento di set-up distribuito in allegato al PSP ha generato un avanzamento

di livello da 504 a 505 per Intel e da 01 a 02 per Olivetti

3-Per migliorare il funzionamento del componente LM78 (verifica dello stato funzionale dell’HW),

in posizione C1M4 e  stato sostituito un condensatore da 22 uF con una resistenza da 30 Ohm

La modifica comporta il passaggio di livello da 505 a 506 per quanto concerne Intel, mentre per

Olivetti il livello e’ rimasto a 02

4-L’Intel ha introdotto un nuovo circuito stampato il quale cambia il proprio livello da

Fab E a Fab F.la mother board assume il nuovo codice Intel AA 669523-604, mentre

il codice Olivetti evolve con un normale avvicendamento da 02 a 03

5-Per lo stesso motivo descritto al punto 3 sul nuovo circuito stampato di mother board, il

componente LM78 revisione C, e’ stato sostituito con la revisione J, questa nuova versione

non necessita più della resistenza da 30 ohm, precedentemente installata in locazione C1M4

pertanto e’ nuovamente stata rimossa.

Con questa modifica il livello HW Intel passa a 605, mentre il codice Olivetti passa a 04

Il BIOS viene aggiornato con la nuova versione 1.00.02.DUO che viene distribuito

in allegato al presente bollettino; di seguito si trova l’elenco dei problemi fissati

Lista problemi chiusi sulle varie versioni di BIOS, fino alla ver. 1.00.02.DUO

- 1.00.02.DU0_ Release 2.0

- LANDesk Service Agent version 1.87 embedded

- S3 86C775/86C785 Video BIOS version 1.01.10-C2.08.05 embedded (V2 DX)

- S3 86C375/86C385 Video BIOS version 1.01.02-C2.05.01 embedded (Virge DX)

Features/Errata Fixed In This Release

Release 2.0

- Removed Trend Micro anti-virus code.

- Core fix for #SERR not being set when Intergraph 3D video card installed.

- Remove memory timing fix which backed off the memory timings when a 64MB or greater DIMM was installed. This was a fix accidentally left in specifically for another product. It could use Fast Page SIMMs and required that if a 64MB SIMM was installed then the timings had to be backed off.

- Fixed memory detection problem with lower density/higher density DIMMs not be sized correctly.

- Release 1.0

- Added LS-120 support.

- Fixed issue with int13 extensions and Win SCT 5.4 failures.

- Corrected issue of a short hard drive access time-out, which was the result of a software            timing loop inside the INT 13H code.  This issue would appear when newer and faster processors are integrated.

- Fixed the problem the APM BIOS incorrectly sets the resume timer if there is an user alarm active.

- Beta 1.05

- Added foreign language support.

- Updated Pentium(r) II L2 Cache Initialization per MD6. Added support for ECC-capable Pentium(r) II and Deschutes latency timing. Fixed problem where machine check exceptions will occur with  (BBL ECC errors) with ECC BSRAMs.

- Added support to enhance the fault tolerant code to address issues (i.e. DMI strings being lost) related to partial erasure of the flash part as a result of power cycling during a flash                update to the VPD area.

- Added support to activate both DMA and PIO mode of operation for ATA and ATAPI devices.  If the device 	does support some sort of DMA (Ultra or MultiWord), Bus Mastering capability will be enabled as well.  This will allow nonPIIX knowledgeable bus mastering drivers to properly operate.

- Setup reports “Pentium(r) II” processor.

- Included fix for Crystal conflict with IBM PnP Token Ring card.

- Included fix for DMI reporting ECC Dimms when none where present in system.

- Modified Crystal CTRL registers default from FF0 to FE8 (hex) to fix Win95 Device Manager conflict.

- Fixes 3 different errors that were occurring in the event logging. The event log full bit was not getting cleared when the log was cleared. Also the Segment was not correctly               set when doing log control functions which caused failures. Last, when looking through the log, we did not limit the search to 255 entries, so if the log was full we hung the system.

- Fixed bug where the BEL apmtest, if executed a second time without intermediate reboot, resulted in the system shutting off the video and hanging until the tester pressed a key to                wake the system back up.  This was caused by the APM resume timer being left enabled after the APM Disconnect call.

- Enhanced the Type 4 Processor version string so that it would automatically determine the Pentium(R) Processor family type to keep the right string in the structure.

- Beta 1.04

- Support for Crystal's CS423X Audio system controllers and codecs.  This is a new code drop from crystal that replaces old code support for any of the Crystal CS423X chips.  The resources ROM for the chips is now downloaded into the chips before the system PnP isolation.

- Added quick mode support for manufacturing mode.  Adjusted memory aspect of the Quick Mode code to perform memory testing as well as initialization in the Manufacturing test environment.

- Added event logging support for Virus Detection.

- Added C0 BIOS Update for Pentium® Pro Processor with MMX™ Technology.

- BIOS Update API changed to set carry flag when an error code is returned for Function 01h, Write BIOS Update Data.

- Added support for DMI block erase for Types 1-3.

- Removed the duplication of  “_DMI_” in the F000 block.

- Adjusted the ECP I/O Range because of 307 errata for WHQL compliance.

- Updated S3 V2DX video bios to version 1.01.10-C2.08.03

- Added changes to setup (see “setup changes” below)

- Changed information reported in DMI (see “other changes” below)

Features Included In This Release

Beta 1.03

- Fixed a problem where potentially a non-shadowable PnP ISA Option ROM will be incorrectly shadowed before boot when another PCI Option ROM is present. Also, fixed a problem where a shadowable PnP ISA Option ROM will always be not shadowed.

- Added changes to setup (see “setup changes” below) including support for an alternate boot sequence.

- Added “Determine System Wakeup” API (see “New Int 15h Extensions” below)

- Added check for “Network” as a boot device before dispatching LSA code.

- Clear the 307 SIO APSR during Soft Power Off.

- Changed information reported in DMI (see “other changes” below)

- Added Secure Flash Circuit Check.

- Fixed a problem where a PCI Option ROM that is resized can cause another PnP ISA Option ROM to not be executed.

- WHQL/PC95A Requirement.  Changed Option ROM code to backfill shrinking Option ROM space with FF's instead of 00s.

- Moved the transferring of the DMI tables into the F000h block from the end of DIM to just before Int 19h.

- Infrared address reported in Bios Data Area.

- Infrared PnP device ID changed to IrDA compatible (PNP0510)

- Updated LANDesk Service Agent version to 1.87 from 1.85

- Added Intel LANDesk® Boot Virus Detection provided by Trend Micro, Inc.

- Beta 1.02

- Updated LANDesk Service Agent version to 1.85 from 1.79

- Added ability to detect which graphics chip is embedded and dispatch correct video bios.

- Added support for PC97 Int 13h Extensions.

- Corrected DIMM Type reporting in Setup.

- Made changes to Setup Defaults (see below).

- Deallocate the Heceta IO ports (0x290-0x297) when Heceta is not stuffed or disabled in Setup.

- Added code to make DRAM timings more relaxed for booting.

- Updates to SIO device nodes to accommodate changes to NS 87307 for improved Win95* compliance.

- Changed Floppy Disk Controller IO Port Allocation from 0x3F2, 0x3F4-0x3F5 to 0x3F2-0x3F5 for improved Win95* compliance.

- Fixed issue where the system would hang during POST if an option ROM switches processor modes and returns the processor back to real mode, rather than Real Big mode, prior to giving control back to the BIOS.

- The Int 15h function 87 (Extended memory block move) is an example that the ROM BIOS may call and would result in a POST time system hang.

- Fixed problem of the 'Memory Error Detection' Setup option sometimes showing up on the Advanced Chipset Configuration screen when non-parity SIMMs are installed.

- Beta 1.01

- Initial Beta

Setup Changes

Beta 1.04

- Added the ability for the administrator to set the privilege level for the user security mode, using the "User Privilege Level".  See “Password Changes” below for a description of the four modes for user mode.

- Changed help text for Alternate Boot Devices.

- Beta 1.03

- Added “Boot Device Options” menu and “Boot Virus Detection” option to “Boot Devices”.  The “Boot Device Options” submenu has the “LAN Power On Startup Sequence”.

- Changed “Power On LAN” default to enabled

- .

- Beta 1.02

- Auto-detect Remote Wake-Up ASIC and display “Power On LAN” option in setup if on board.

- Changed “Power On LAN” default to disabled.

- Changed “Power On LAN” and “Power-On COM1 Ring” to not be hidden if “Auto Start On AC Loss” is disabled.

Password Changes

Beta 1.04

- Added the ability for the administrator to set the privilege level for the user security mode, using the "User Privilege Level". There are 4 modes for the user mode:

- Limited Access - Here the user can change date, time, user password, Unattended Start, the Security Hot key.

- Full Access - Here the user can access everything except Admin. Password and the "User Privilege Level".

- View Only - Here the user can view all fields available, but not change anything and can only Exit Discarding Changes.

- No Access - Does not allow the user to access setup.  Added the ability for the administrator, or the user in full access mode, to clear the user password via the "Clear User Password" field, without knowing what it currently is set to.

Language Support

Beta 1.01

- English only

Hard Disk Changes

Plug and Play (Windows 95, ISA Configuration Utility, etc.) Changes

APM  changes

New Int 15h Extensions

Beta 1.03

- Int 15h - Function 2307, Subfunction 5755, Determine System Wakeup API

Description: This function returns a value representing which event caused the

system to power up.

Input:  AX  =   2307h

BX  =   5755h

Output: CL  =   06  Power switch

05  LAN Remote Wakeup

04  COM 1 Modem Ring

03  Reserved for APM Timer wakeup

NC  =   No error

Other Changes

Beta 1.04

- Updated DMI Type 4 processor version string and socket designation.

- Beta 1.03

- DMI reports 3.3 volts for memory module Type 5.

- DMI reports 5 volts for ISA slot 5 in Type 9.

- DMI now correctly reports if ECC/Parity DIMMs installed.

- DMI reporting corrected for DIMM RAS/CAS lines and reference designators for audio line out, audio mic in, and CD audio.

Known Errata With This Release

Features Still To Be Added / Comments About This Release