| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V. Leva |  |  | A. Niosi |  |  |  |  |  |  |  | May-96 |  |  |  |  |  |  | 2800227D |  |  |  |  | 02 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| PC |  |  | M4-P75i-100i-133i-150i |  |  |  |  |  |  |  | Motherboard |  |  |  |  |  |  | ATLANTIS |  |  |  |  |  |  |  |
| titolo / subject ––––– –BIOS 1.00.04.CL0–––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  |  |  | - altri / other ––––– |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time ––––– |  |  |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level ––––– |  |  |  |  | livello minimo di aggiorn. minimum revision level ––––– |  |  |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autoriz. per riparaz. in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |  |  |

SUBJECT:	Atlantis Bios Release 1.00.04.CL0

Features/Errata Fixed In This Release

- Updated the Crystal  firmware (audio).

- The firmware support is moved to Plug-N-Play code.

- Reinstated the Hard Disk Pre-Delay feature.

- Fixed problem where CPU Auto HALT features were incorrectly being disabled.

The fix results in about 5 watts of additional power saved during CPU inactivity even when APM is disabled in setup as long as a power driver is installed).

- Added call to detect serial mouse if a PS/2 mouse is not installed in the system.  Will now wake the system if a serial mouse is installed and a PS/2 mouse is not installed.

- Fixed problem in which not all resources disabled if all resources cannot be enabled for a PCI device.

- Fixed timing loop in keyboard controller related code that was causing problems with network remote diskless boot.

- Set Input attribute in Keyboard device node.

- Set IPL attribute in Floppy and IDE device nodes.

- Corrected IRQ programmed for 3F8,IRQ 3;  2F8,IRQ 4;  3E8,IRQ 3; & 2E8,IRQ 4.

Programming IRQ 3 & 4 reversed.  3F8,IRQ 4; 2F8,IRQ 3; 3E8,IRQ 4;& 2E8,IRQ 3 were programmed correctly.

- Corrected problems and potential problems possibly causing CMOS corruption.

- Battery failure status was not being preserved through POST.

- Date and time could be set to default (01/01/90) without posting an error.

- Accesses to enable or disable NMI were not followed with a read or write.

- Re-enabled palette snooping for those PCI BIOSs that disable it during initialization and request palette snooping be enabled via setup.o

- Corrected floppy function 08, Return Disk Parameters.

It returned wrong parameters if the drive number was illegal.

- Fixed bug in which BIOS text string could be displayed incorrectly.

- Disabled Hotkey 6 in the Phoenix Keyboard Controller.

It's default is enabled and set to the Break Key.

This caused problems with some software.

- Added an additional test on the boot record (test for 0000 at the start of the record) to avoid trying to boot from a non-system disk.

- Updated the ATA/ATAPI detection code to be more robust.

- Handles faster CPUs, more reliably detects CD-ROM drives,and handles IDE drives which act as master and slave when no slave exists.

- Fixed Int 15 Function C7h where bogus amount of memory could be returned.

- Fixed password problem in which the password is entered followed by a backspace is considered valid.

- Fixed possible Gate A20 enable/disable race condition.

- Replaced current video BIOS with ATI264-CT DUAL CAS 60Ns RAM 54 MHz Video BIOS version 1.073/54M.

This video BIOS is needed to change the internal device  mem clock to 54 MHz to correct video DRAM timing problems.

- Fixed problem where CPU Auto HALT features were incorrectly being disabled.

The fix results in about 5 watts of additional power saved durring CPU inactivity even when APM is disabled in setup (as long as a power driver is installed).

Features Included In This Release

- Reinstated the Hard Disk Pre-Delay feature.

- Added call to detect serial mouse if a PS/2 mouse is not installed in the system.

Will now wake the system if a serial mouse is installed and a PS/2 mouse is not installed.

- Replaced current video BIOS with ATI264-CT DUAL CAS 60Ns RAM 54 MHz Video BIOS version 1.073/54M.

This video BIOS is needed to change the internal device mem clock to 54 MHz to correct video DRAM timing problems.

Setup Changes

- Reinstated the Hard Disk Pre-Delay feature

Language Support

- US, IT , GR, FR, SP

Hard Disk Changes

- Updated the ATA/ATAPI detection code to be more robust.

- Reinstated the Hard Disk Pre-Delay feature

APM 1.1

- Added call to detect serial mouse if a PS/2 mouse is not installed in the system.

Will now wake the system if a serial mouse is installed and a PS/2 mouse is not installed

- Fixed problem where CPU Auto HALT features were incorrectly being disabled.

The fix results in about 5 watts of additional power saved durring CPU inactivity even when APM is disabled in setup (as long as a power driver is installed).

New Int 15h Extensions

- Fixed Int 15 Function C7h where bogus amount of memory could be returned.