| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V. Leva |  |  | A. Niosi |  |  |  |  |  |  |  | Feb-97 |  |  |  |  | 2800297L |  |  |  |  | 03 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| PC |  |  | MODULO xxx SL-DT-MT |  |  |  |  |  |  |  | Motherboard |  |  |  |  | BA 2292 - BA 2322 |  |  |  |  |  |  |  |
| titolo / subject ––––– NEW BIOS 1.00.10.CV2 ––––– ––––– ––––– ––––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  | - altri / other ––––– |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time ––––– |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level ––––– |  |  |  |  | livello minimo di aggiorn. minimum revision level ––––– |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autorizzati per riparazioni in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |

BIOS 1.00.10.CV2

Features/Errata Fixed in this Release

- Fixed issue where single bit ECC errors could cause the system to either lock up or in the case of Windows NT, cause a Kernel Dump.

- Added a check to prevent the parity/ECC event log handler from incorrectly logging errors when parity/ECC checking is disabled. This corrects the case where parity/ECC errors were being logged for non-parity/ECC memory modules.

- Fixed issue where the system would hang during POST if an option ROM switches processor modes and returns the processor back to real mode prior to giving control back to the BIOS.  The Int 15h function 87 (Extended memory block move) is an example that the ROM BIOS may call and would result in a POST time system hang.

- Added new ATAPI code which allows system to boot the WinNT

- 4.0 CD and other CD’s with similar formats.

Fixed manual peripheral configuration (Serial and Parallel Ports).

Beta 10.01

Added additional checks for displaying the DMI VPD info during POST.  Now if no serial number or Product name is programmed the BIOS will not attempt to display these fields.

Added additional DMI support for PCI slot reporting. Fixed bugs in support for enhanced PCI latency timer code.

Fixed code to preserve CMOS scratch byte when defaults are loaded so that INT15 DA29 works correctly.

Added support for CD-ROM as boot device other than first.

Closed window between CMOS write and CMOS checksum to reduce possibility of a power down during POST causing a CMOS checksum error.

Fixed a CMOS ChkSum fail (Reg 0Eh) related booting problem.

Shadow FFhs from E0000h to E7FFFh, if possible, so EMM programs can use area for UMBs. Also for Microsoft WIN96 logo purposes.

Fixed National 306 Get Node problem.

Added CMOS entries for EPP1.9 & ECP/EPP1.9 & associated code.

Corrected problem of FMUP Verify failure on boards with Event Logging.

Modified the Latency Timer code to allow an intelligent method for determining the actual value to be programmed into each PCI device's timer register.

Use new ATI A4 video BIOS version 3.008

Release 9.0

Disabled USB.

Release 8.0

Relaxed RW burst timings to x-3-3-3 when ECC enabled

Beta 8.02

Enabled Delayed Transactions.

Added support for ECC Event Logging.

The 32 clock retry feature is disabled for ECC Systems.

Added support for Mfg. DMI utility to display the board name and the serial number on bootup.

MMX Overdrive reporting.

Beta 8.01

Added more boundary checking to INT 13.

Fixed bug in software delay in 32 bit mode.

Changes for WIN95 supplement LOGO acceptance tests.

Fixed issue where single sided EDO 16MB simm pairs were reporting EDO for one SIMM and FP for the other.

Fixed Issue that DMI will always report cache as enabled.

Fixed bug where PnP cards could get memory from E0000 to FFFFF.

Incorporated Event Logging which includes POST Errors and Extended POST Errors.

Fixed problem of system hang during ESCD clear using third party 'Clear ESCD' program.

CPU frequency is detected earlier in POST to satisfy DMI requirements.

Fixed problem w/ parity errors.

Corrected situation where ESCD could not be updated when flash block was left in an indeterminate state by an erase.

Corrected APM code to blink power LED during power down.

Enabled USB.

Release 7.0

Include both A3/A4 video BIOS’s and auto-detect which revision is present and load the appropriate video BIOS. A3 - Revision 2.118, A4 - Revision 2.221

Included DMI fixes in structures (corrected values).

Included DMI CPU speed fix.

Removed USB enabling. It is now disabled.

Fixed problem with dynamic disabling of PS/2 mouse in Win95.

New Creative Vibra 16S/C audio detection and initialization code.  (See “Other Changes” section at the bottom of this document for a description)

Beta 7.02

New ATI-A4 Video BIOS (version 2.221)

Beta 7.01

Fixed APM 1.2 issues with Microsoft.

Fixed manufacturing defaults to comply with 4.20 spec.

Added new video BIOS (A4), version 2.198

Fixed issue with certain PCI SCSI cards. During device (PCI) initialization, the DIM buffer is now above 1MB and not below 640KB. So PCI ROMs which use this area for setup vector don’t conflict with the system BIOS.

Fixed ACFGTEST XREF failures with PXII3 device.

Release 6.0

Included an errata for a silicon bug. Snoop ahead during PCI read cycles has a bug. During testing a specific scenario was discovered that may cause incorrect data to be forwarded to a PCI master during a read cycle.

Reporting of 200mhz CPU speed was corrected.

Beta 6.03

Enabled USB and display “USB Development Kit” during signon.

Beta 6.02

Included in the latest DMI fix. Strings passed to DMI BIOS calls are now checked for null terminators.

CD ROMs on the secondary controller which were not being identified are now being identified correctly. IRQ routing method was changed.

Beta 6.01

The memory detection fix was modified again. The cause of the problem was write buffering and the new routine flushes the buffer on writes.

70ns Memory Timing values were corrected.

ECC Memory Timings were corrected.