| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V, Leva |  |  | A, Niosi |  |  |  |  |  |  |  | Apr-97 |  |  |  |  | 2800297L |  |  |  |  | 06 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| PC |  |  | MODULOxxx SL-DT-MT |  |  |  |  |  |  |  | Motherboard |  |  |  |  | BA2292 BA2322 BA2324 BA2337 |  |  |  |  |  |  |  |
| titolo / subject NEW BIOS 1.00.12.CV2 ––––– ––––– ––––– ––––– ––––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  | - altri / other ––––– |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time ––––– |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level ––––– |  |  |  |  | livello minimo di aggiorn. minimum revision level ––––– |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autorizzati per riparazioni in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |

BIOS 1.00.12.CV2

Features/Errata Fixed In This Release

Release 12.0

- Modified code to enable USB if the USB manufacturing option is present on the board.

Release 11.0

- Added support to enable the USB hardware. The setup option to enable/disable USB will appear in the peripheral submenu. If the board does not support USB hardware then the option does not appear in BIOS setup.

- Reserved the area of the system BIOS flash space ((0fff80000h-0fffbffffh) (4GB - 512KB)) for improved Windows 95* compliance.

- Modified flash tables to limit logo area to 4KB.

BIOS Upgrade Instructions

-------------------------

Creating a Bootable Floppy (You must use a DOS system to create the

bootable floppy)

--------------------------------------------------------------------

1.  Place an unformatted floppy diskette in the floppy drive and format

the floppy using the /S option.  example: "format a: /s"

2.  Alternatively, place a formatted floppy in the floppy drive and use

the "sys" command.  example: "sys a:"

Upgrading to the Latest BIOS

-----------------------------------

1.  Be sure you have downloaded the correct BIOS file from the BBS.

2.  Note the settings of the SETUP parameters.  Enter SETUP by hitting

the F1 key during boot up.  Write down the settings for each of

parameters.  At the end of the BIOS update process you should set the

parameters to default values by hitting the F5 key, and then

re-enter these values you have written down.

3.  Follow the instructions (above) for creating a bootable floppy to

create a bootable floppy diskette.

4.  Insert the bootable floppy into drive a:.

5.  Extract the contents of the BIOS file you downloaded, onto the

bootable floppy.  You simply type the filename (without the

extension) followed by a: , for example "BIOS A:".

6.  Place the bootable floppy containing the BIOS into drive a: of the

system that you want to upgrade and boot the system while the floppy

diskette is in the drive.

7.  Press ENTER to go to the Main Menu.

8.  Highlight the option that reads "Update Flash Memory From a File" and

press ENTER.

9.  Highlight the option that reads "Update System BIOS" and press

ENTER.

10. At the screen that asks you to enter the path and name of the file,

press TAB to highlight the file name and press ENTER.

11. The system will give you a warning that BIOS is about to be changed.

If you wish to continue simply hit ENTER.

12. Once the BIOS has been successfully loaded, remove the floppy

diskette from the drive.  Hit ENTER at the prompt to reboot the

system.

13. Check to make sure the BIOS version is the new version as the system

reboots.

14. Enter Setup by pressing the F1 key during boot up.  Once in Setup,

press the F5 to set the parameters back to default values.

Re-enter the values you wrote down at the beginning of this process.

If you do not set the CMOS values back to defaults using the F5

key, the system may function erratically.

Note: you may encounter a CMOS Checksum error or other problem after

reboot.  Try shutting down the system and booting up again.  CMOS

checksum errors require that you enter Setup, check your settings, save

your settings, and exit Setup.