| compilato da / compiled by |  |  | approvato da / approved by |  |  |  |  |  |  |  | data / date |  |  |  |  | codice - progressivo / code - number |  |  |  |  |  | TIB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M.Fontana |  |  | M.Merlone |  |  |  |  |  |  |  | Jan-97 |  |  |  |  | 2800267H |  |  |  |  | 05 |  |  |
| classe-prodotto / class-product |  |  | sistemi interessati / systems concerned |  |  |  |  |  |  |  | area-funzione / area-function |  |  |  |  | piastra-gruppo / board-assy |  |  |  |  |  |  |  |
| SNX-Netstrada |  |  | Netstrada 3000 |  |  |  |  |  |  |  | ––––– |  |  |  |  | ––––– |  |  |  |  |  |  |  |
| titolo / subject ––––– ––––– Technical informations about Netstrada 3000 System ––––– ––––– ––––– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | - variazione di prestazione - feature change - miglioram. di producibilità - production improvement |  |  |  |  | - riduzione di costo - cost reduction - migliore assistibilità - servicing improvem. |  |  |  |  |  | - miglioramento qualità e affidabilità - improvement in quality and reliability - variazione di software - software change |  |  |  |  |  | - altri / other General Informations |  |  |  |  |
|  | matricole / serial number ––––– |  |  |  | livello iniziale / initial level ––––– |  |  |  |  | luogo e tempo implementation & time ––––– |  |  |  | applicazione / implementation procedure |  |  |  |  |  |  |  |  |  |
|  | data applicaz. in produzione date introduced into production ––––– |  |  | livello finale / final level ––––– |  |  |  |  | livello minimo di aggiorn. minimum revision level ––––– |  |  |  |  |  | - per la sola informazione - for information only - 1° rientro - 1st return - malfunzionamento - on fault |  |  |  |  | - sistemat. in modo prevent. - mandatory retrofit - 1° intervento - 1st service - centri autorizzati per riparazioni in garanzia - warranty authorised repair centres |  |  |  |
| pubblicazioni interessate publications concerned ––––– |  |  |  |  |  |  |  | codice aggiornamento code and updating level ––––– |  |  |  |  |  | note notes ––––– |  |  |  |  |  |  |  |  |  |

Please, find attached technical informations regarding Netstrada 3000 System.

A detailed description and the limitations/warnings present at the start-up phase are also reported.

# Table of contents

1. Table of contents

2. FOREWORD

3. System Overview

3.1. Motherboard Architecture Overview

3.1.1. Block Diagram

3.1.2. Pentium Pro Processor

3.1.3. PMC and DBX

3.1.4. PIIX3

3.1.5. PCI I/O Subsystem

3.1.6. ISA I/O subsystem

3.1.7. DRAM Array

3.2. SCSI Subsystems

3.2.1. Removable Devices

3.2.2. Hard Disk Units

3.2.3. SCSI interconnection

3.3. Power Supply

3.4. Rack Mount kit

4. CONFIGURATION FOR RELEASE

4.1. Bios / User Disk / System test

4.2. STARTER KIT CODE

4.3. STARTER KIT UPGRADE

4.4. PDG RANDOM CODE

4.5. SYSTEM  BOARDS

4.6. Operating System

4.7. Hardware configuration

4.8. SCSI Controller

4.9. SCSI configuration

4.10. Tested Network Card

5. Warnings & Limitations

# 2-FOREWORD

This document is aimed at describing the status of Hardware, Software, Bios, User Disk and Diagnostic.

The  system is released with the following operating systems:

- NT3.51;  NT 4.0

- Novel Netware 3.12; 4.11;4.1

- IBM OS/2 Warp Server; 2.11

- SCO 5.02;

- SCO Unixware 2.1

The system is released both in Mini Tower  (Box Copper) and Rack configurations.

# 3-System Overview

The motherboard is an Intel ESG (Enterprise Server group) design named “R440FX UP Server" and nicknamed "Rosewood”, that provides the following features:

- Single Pentium Pro processor

- Processor speed support for 166, 180, 200 and 233 MHz. Only 200 MHz part will be certified .

- 82440FX “Natoma” chipset

- separate 32 bit address/ 64 bit data path

- Four single module ECC DRAM banks hosted into four 3.3 V DIMM sockets supporting both FPM (Fast page Mode) and EDO (Extended Data Output) memory protocols

- Memory addressing capacity up to 1 GB (512 MB max. supported at product launch)

- Up to six PCI devices rev. 2.1 compliant (5V signaling environment only) including:

- ULTRA SCSI (AIC7880) on motherboard

- Cirrus 54M40 video controller on motherboard

- Intel PRO/100 10-100 Mb compatible LAN controller on motherboard

- 3 PCI slots for further expansion

- ISA I/O system with 2 expansion slots

- Embedded PC-compatibility I/O support (2 serial, parallel, mouse and keyboard)

- Server management features, including thermal/voltage monitoring and error handling supported by Server View

- System Management Module (SMM-Hobbes) connector

- Flash BIOS support for all the above

- Extended ATX form factor

The box Copper will provide the following features:

- Extended ATX cabinet

- Up to four 1” hot pluggable HDU organized either on a single or a dual bus (duplexing)

- CD-ROM 8x always installed in the basic unit

- One more 5”1/4 half size removable SCSI bay for DAT or Streamer

- 180-200 W ATX power supply

- On-off front panel push button with key lock

- HDU bays door key locked

- LED reporting for: system on, SCSI activity, SCSI fault, System fault

- I/O section dedicated fan for improved system cooling

- Rack mount kit to mount the server in the Olivetti “SPACE 2000” cabinet line or on compatible 19” rack cabinets (5U high)

- Up to two external PEM 400 cabinets can be used to expand system disk capacity (RAID controller required)

## 3.1-Motherboard Architecture Overview

This chapters provides an overview of the R440FX UP motherboard showing functional blocks and their relationship and board layout diagrams.

The motherboard architecture is a single board solution, based on Pentium Pro microprocessor and Intel 440FX chipset. The system places the microprocessor and ECC memory, PCI and ISA I/O systems (with embedded devices for video, network, and disk control) on the motherboard. Following is a list of featured subsystem and devices:

- Single ZIF socket 8 for the installation of the Pentium Pro processor

- Onboard DC-to-DC converter for processor power

- GTL+ termination for processor bus signals

- PCI and Memory Controller (PMC) and data Bus Accelerator (DBX) from 440FX chipset

- Support for up to 1GB of DRAM using 64M-bit technology mounted on JEDEC 3.3V buffered 60ns DIMM

- PCI to I/O system with 3 expansion connectors and these embedded devices:

- PCI/ISA/IDE Accelerator (PIIX3 from 440FX chipset) with PCI-to-ISA bridge, PCI IDE interface (not used on NetStrada3000), and Universal Serial Bus (USB) controller functions (not used on NetStrada3000)

- PCI video controller

- SCSI controller

- Network controller

- ISA I/O system with 2 expansion connectors

- National Semiconductor Super I/O chip providing all PC-compatible I/O

- Interrupt control and system management interrupt support

- Server management/monitoring support, and I2C bus providing an independent communications path for onboard server management controllers and dumb devices.

- Connector for Hobbes board

### 3.1.1-Block Diagram

.

### 3.1.2-Pentium Pro Processor

There is one socket-8 installed for the Pentium Pro processor featuring Zero Insertion Force connection for easy processor upgrading. The socket also provides notches for heatsink clips.

The processor is powered by an onboard voltage regulator that can provide adequate voltage to the processor based on CPU VID configuration pins so that the processor can be upgraded with newer parts that may require different voltage range with automatic power supply reconfiguration and no manual intervention is required.

The processor is located in the corner closest to external I/O connectors as specified by ATX specs so that it can receive an adequate air flow from the power supply on board fan and has a 1” passive heat sink to accomplish the specified cooling.

A specific fan connector for active heatsink is available should it become necessary.

### 3.1.3-PMC and DBX

Two devices from the Intel 440FX chipset, PMC and DBX, form the heart of the NetStrada3000 motherboard. These devices implement the pathway between Pentium Pro processor, memory and PCI I/O. As the PCI-host bridge, this path maintains proper ordering of operations, trapping synchronization events and flushing buffers appropriately. The PMC and DBX support a 32 bit address and 64 bit data Pentium Pro processor interface , which operates at 66 MHz (or 60 MHz, depending on the processor core frequency) in the GTL+ signaling environment.

The PMC supplies a 32 bit PCI interface which operates synchronously at half the processor bus frequency (thus either at 33 or 30 MHz), in a 5V signaling environment.

The PMC also acts as memory controller for the system, with the DPX providing the data path for the memory. The memory controller supports 1 GB of ECC memory, using Fast Page Mode (FPM) or Extended Data Out (EDO) DRAM, on commodity DIMM devices.

The PMC and the DBX communicate using a private control and data bus, which operates independently of transactions occurring on processor and PCI buses.

### 3.1.4-PIIX3

One of the devices in the 440FX chipset, the PCI/ISA/IDE Accelerator (PIIX3) is a multifunction PCI device . There are three distinct PCI controllers on the PIIX3: PCI-to-ISA bridge, PCI-based fast IDE interface and USB controller. Also, ISA subsystem features on the device includes a Plug-n-Play port, system timer, compatibility interrupt controllers, enhanced DMA functions, system power management support, and I/O APIC decode support.

The IDE interface, USB interface and I/O APIC functions are not used on the R440FX UP Server board.

### 3.1.5-PCI I/O Subsystem

The primary I/O bus for the R440FX UP Server card is PCI, compliant with revision 2.1 of the PCI specification, and operating at up to 33 MHz. The routing of PCI signal is tightly controlled to deliver the highest amount of embedded functionality possible for a single PCI segment: 3 expansion slot, PIIX3, video controller, SCSI controller and network interface controller.

#### 3.1.5.1-PCI video controller

The R440FX UP Server card uses the next generation of integrated video controller and support circuitry: Cirrus Logic CL-GD54M40 32-bit VGA Graphics Accelerator chip containing an SVGA video controller, clock generator, and 80 MHz RAMDAC in a 160 pin PQFP. It comes equipped with 1Mbyte video RAM installed.

#### 3.1.5.2-SCSI subsystem

The cards provides an embedded SCSI host adapter: Adaptec AIC-7880 (Katana). The AIC-7880 features support for ULTRA SCSI, WIDE SCSI and full-featured PCI bus master interface in a 160 pin PQFP. As a PCI 2.1 bus master the AIC-7880 supports burst data transfers on PCI up to the maximum rate of 133 MB/sec using the on-chip 256 byte FIFO.

#### 3.1.5.3-Network controller

The R440FX UP Server supports a 10BASE-T/ 100BASE-T network subsystem based on the Intel 82557 Fast Ethernet PCI Bus Controller. National DP838040 and DP83223 devices, along with other discrete components, provide the physical layer interface. As a PCI muster, the 82557 can burst data at up to 133 MB/sec

### 3.1.6-ISA I/O subsystem

The ISA I/O subsystem on the R440FX UP Server provides 2 ISA slots and embedded compatibility I/O. The ISA bus also supports the Flash BIOS, and a Server Management Controller with host ISA interface ASIC. A National Semiconductor Super I/O chip (87308VUL) resides on the ISA bus, which supports direct connection of Keyboard/ Mouse, Floppy drive, and standard parallel and serial ports.

### 3.1.7-DRAM Array

The memory controller can automatically detect and initialize the memory array (non manual intervention for jumper setting is requested), depending on the type, size and speed of installed DIMM devices.

Memory is partitioned into four non interleaved banks, one 3.3V DIMM each, each one providing 72 bits: 64 bits for the data and 8 for ECC.

DIMM sockets can be populated in any order, mixing DIMM size and types if necessary. Total installed memory is automatically sized and allocated by the PMC and reported to the system. NetStrada3000 will be introduced with the support of FPM DIMM with size of 2M72 (16 MB), 4M72 (32MB), 8M72 (64 MB) and 16M72 (128 MB). EDO DIMM and larger FPM DIMM (like 32M72 (256 MB)) will be introduced as they will become available or necessary.

System memory begins at 0 and is continuous (flat addressing) up to the maximum amount of DRAM installed. The only places where system memory is not contiguous is in the ranges defined as memory holes using configuration registers.

## 3.2-SCSI Subsystems

As every server in Olivetti offer, the mass storage is connected through SCSI interface, with the only exception of the FDU. Within the SCSI subsystem two different class of devices are addressed: HDU in on class and removable devices (CD-ROM, DAT, STU) in the other.

Both this system have a SCSI interface and can either be connected to the same SCSI controller or to two separate controllers.

### 3.2.1-Removable Devices

This class covers CDROM, DAT and STU and there are two 5”1/4 HH bays available in the system chassis to host them. Although they might be combined in any configuration, it has been decided to have always the CD-ROM installed in bay 2 (the lowermost) and bay 1 (the uppermost) free for customer needs.

The SCSI CD-ROM is an 8x device with a 50 pin SCSI connector. It is ALWAYS located as last device on the removable SCSI cable and therefore MUST always have its internal terminators enabled. The device has been chosen so to feature active termination to enable disk to transfer data in compliance with ULTRA SCSI specs in the configurations which have both removable and disk devices connected on the same SCSI channel.

Any DAT and STU can be installed on the system as long as they present with a 50 pin SCSI connector and fit mechanical dimension of the bay. The front panel might be smaller than the full bay size (typically with DAT) and, in this case, the bay blank filler used to cover the bay when there is no device inserted, can be partially broken along a predefined knock out section and reused to accommodate the device providing a 5”1/4 compliant, nice looking front bezel.

This technique  has already been successfully used with other Olivetti cabinets, like the ”Silver”.

The list of all supported and certified removable devices for NetStrada3000 will be presented in a separate document.

All the removable units need to have their ID set at installation time. There is no restriction to the ID they can have with the exception of ID7 which is reserved for the controller. However if they are connected to the same SCSI bus as the hard disk, care should be taken that the HDU already have ID0 to ID3 assigned.

### 3.2.2-Hard Disk Units

The NetStrada3000 will only support hot swap HDU that needs to be mounted on a specific tray to be inserted in the chassis. With this definition we just refer to the mechanical assembly of the units, to have a real hot swap feature is also necessary to have them connected to the RAID controller. If they are connected to any other controller (either the motherboard SCSI controller or any other SCSI controller) they cannot benefit of hot plug and should not be removed for any reason unless the system has been turned off.

The chassis can host up to four units located in front of the system and protected by a key locked door. The four bays have self ID with ID0 starting from the uppermost unit down to ID3 for the lowermost one.

The four bays are all able to host 1” HDU and the system will be released with disk capacity of 1, 2 and 4 GB.

To be compliant with hot plug requirements all the disk must bear an 80 pin SCA connector to plug into the SCSI backplane.

The disk can either be 8-bit or WIDE SCSI capable of transfer rate of  10 (FAST) , 20 (Wide) e 40 (ULTRA Wide) Mtransfers/sec (only during data phase). The disk certified for the system will however always be ULTRA WIDE SCSI.

Mechanical supports and shock absorbers have been designed so that the chassis can host both 3600, 5400 and 7200 rpm disk types. Certified disk will however all feature either 5400 or 7200 rpm.

### 3.2.3-SCSI interconnection

SCSI interconnections have always been a complex issue due to the many different topologies and configurations allowed. The issue has been recently further complicated by the introduction of the 80 wire WIDE SCSI for the disks while the removable still have a 50 wires cabling. Moreover more than one controllers can be present at the same time giving way to configuration proliferation.

In this section we will introduce the basic cabling scheme for entry level systems and the rules to expand them to more complex configurations. An entry level system from a peripheral standpoint, is one that has just the CD-ROM, any number of disk from 1 to 4, and just the embedded motherboard SCSI controller.

The cabling for such a configuration is based on a mix of flat cables and PCB backplanes. All this parts will also be present in more complex configurations and all of them can be implemented just connecting them differently. The only exception to this rule is the controller duplexing which requires a separate kit with an extra cable.

Basic system cabling is illustrated in the following diagram.

Figure 1: SCSI Cabling Diagrams

The board and the cables shown in the figure are always present in every configuration even though the connection may slightly vary.

The boards functions are the following:

- IF2060: SCSI back plane. It provides direct attachment to the four disk via 4 SCA-2 connectors and the relative data path. Signal through connectors are routed one every other connector (ID0, ID2, Jumper connector, ID3 and ID1) so to respect the minimum stub length suggested by ULTRA SCSI specification. In duplexing configurations, the data path is split in two halves, one connecting the odd IDs, the other the even IDs.

The back plane also carries two LEDs every disk: a green one for the disk select and an amber one for disk failure. See RAID controller specification for details on their usage.

It has to piggy backs: the IF2061 swap board and the IF2054 Jumper board (or, alternatively, the IF2053 Terminator board for duplexing configurations).

- IF2061: Swap board. As in every Olivetti hot plug subsystem, the back plane also carries a swap board that is in charge of managing the fault bus and control the disk fault LED (amber). It also carry the active terminators for the SCSI channel (or for the 2nd SCSI channel in case of duplexing),  and provide proper signal routing between the motherboard Front Panel Connector and the on/off push button and the front panel LEDs.

- IF2054 Jumper board. This is just a 1 to 1 interconnection between the odd and the even halves of the back plane bus so that it can be logically addressed a s a single bus.

- IF2053 Termination Board. It replaces the IF2054 Jumper board in duplexing configurations. It provides a WIDE SCSI connector for the second SCSI cable that will be connected to the back plane odd half, and provide active termination for the even half. It will come in a specific duplexing kit that is not included as standard in the box.

- IF2063 WIDE/Narrow ”T” adapter. The motherboard provides a WIDE SCSI connector only and this piggy back just allow two different connections: a WIDE one and a “NARROW” one. It comes standard in any box.

Two cables are also present in the box: a two connector, point to point WIDE SCSI cable (blue in the figure)  and a three connector 50-wires cable (cyan).

In case of different interconnection topologies, the following guidelines should be followed:

- The WIDE cable should always used to connect the disk back plane either to the onboard SCSI controller, or to the RAID controller, or to any other supported controller

- The “NARROW” cable should always be used to connect the removable devices with the CD-ROM to one end. The CD-ROM should always have terminators enabled.

- SCSI termination should select according to the following rule:

  - The CD-ROM is always terminated and terminates one end of the bus. However it terminates only the lower 8 bit of the WIDE SCSI bus and, if disk and removable are connected on the same bus, the upper 8 bits need to be terminated on the controller (motherboard controller in the figure)

  - The SCSI back plane is always terminated

  - If the controller is connected to either WIDE or NARROW SCSI cables (but not both), it should have both upper and lower bus terminators enabled. For the onboard controller this selection is accomplished with the Adaptec configuration utility the can be invoked with <CTRL A> at POD.

  - If the controller is connected to both WIDE and NARROW cables, it should have the upper byte terminators enabled and the lower byte terminators disabled. Again, with the onboard controller this can be selected with the configuration utility.

## 3.3-Power Supply

The system is powered by a standard ATX power supply.

## 3.4-Rack Mount kit

The system can be rack mounted with the use of a specific kit that will make it compliant with 19” standards. While all other Olivetti systems so far had a specific chassis for rack mount, the NetStrada3000 system has an add on mechanical kit that is assembled over the standard box.

To have it installed, it is necessary to remove the cabinet top cover, the four feet and the front door. The system can than be laid on its right side and rack front panel, side panels and slides can be assembled.

The system will be 5U high.

# 4-CONFIGURATION FOR RELEASE

## 4.1-Bios / User Disk / System test

No major problem has been reported for bios, user disk, the released versions

are the following:

- Bios 1.00.02.DM0

- User Disk Diagnostic 1.0 update 2

- System Test	         1.0

- Configuration Utility 1.0

- Netstrada3000 Bios Extended code  1.00 (final version 1.01 will be released on January 6)

- Netstrada3000 Set Default Utility  1.0

## 4.2-STARTER KIT CODE

The starter kit code is:   213654E-01.

It includes the following :

- CDROM Orchestra rel 2.1  ( cod 2692215R)

- User disk diagnostic (cod 2692216V)

- Getting Started  (cod 2692556F)

- Last Minute Information (cod 2692560S)

## 4.3-STARTER KIT UPGRADE

This kit is released in order to upgrade Netstrada 3000 shipped till January 1997

This Starter kit is mandatory for managing SAR, Duplexing, Resilience, RAID and Server View features.

The starter kit upgrade code is:   214711Y.

It includes the following :

- CDROM Orchestra rel 2.1  ( cod 2692215R)

- User disk diagnostic (cod 2692216V)

- Getting Started  (cod 2692556F)

- Last Minute Information (cod 2692560S)

- Netstrada 3000 set default utility (cod 2692176Z)

- Netstrada 3000 BIOS and Flash Eprom utility (cod. 2692213Y)

- Netstrada 3000 BIOS Extension code and flash eprom utility (cod 2692214M)

- Upgrade User Guide

This Starter kit upgrade may be obtained by shipping the coupon inserted in the starter kit of the system shipped till January 1997 to the address reported on the coupon itself.

## 4.4-PDG RANDOM CODE

Below are listed some of the PDG random code released , for all the code released see Progetto di Gestione rev 10C.1.

- NP30200 A4674D    (empty basic unit)

- NP30200x32x0  A4675E

- NP30200x32U0  A4676F

- NP30200x32U0xxS  A4677 G

- Rack mount kit  A3994N

- DCR PCI1/3 UW  A3976 T  ( DPT UW raid controller)

- SCC PCI 3000UW  A5408D ( Adaptec 2940UW SCSI board)

- DUP KIT3000UW  A3979E (Duplexing Kit)

- PEM 241W/RS      A3987 P

- PEM 240W/RM    A6114 N

## 4.5-SYSTEM  BOARDS

NetStrada 3000  is released with the following  boards:

- Motherboard                   BA 2315    cod. 212794S

- SCSI WIDE Back panel   IF 2060     cod. 212771A

- Swap Board                     IF 2061     cod. 212781V

- Terminator Board             IF 2053     cod. 212653C   (for duplexing configuration)

- Jumper board                    IF 2054     cod. 212654D

- SCSI mux wide-narrow    IF 2063    cod.  562145W-00

## 4.6-Operating System

The following O.S. have been fully certified:

NT 3.51 (service pack 5); NT 4.0

NETWARE  4.1  (with the following patches : 410.pt3;  410.it6;  landr4.exe)

NETWARE 4.11

NETWARE  3.12 (with the following patches: 312.pt7)

OS/2 WARP SERVER ; 2.11

SCO Open Server 5.02

SCO Unixware 2.1

## 4.7-Hardware configuration

The hardware configuration released is as follow:

Pentium PRO 200Mhz/256K cache

DIMM Modules : Fast Page Mode 1Mx72; 2Mx72 ;4Mx72 ;8Mx72 ; 16Mx72

HDU  : 2Gbyte UW  Seagate ST32171WC 7200 RPM 1”

4Gbyte UW  Seagate ST34371WC 7200 RPM 1”

9Gbyte UW  Seagate ST19171WC 7200 RPM 1.6” (Only for Pem RS)

Video Ram : 1Mbyte

ISA Stallion

CDROM  SCSI 8x Panasonic

DAT DDS2

STU 1Gbyte

## 4.8-SCSI Controller

The SCSI controllers released are :

- DPT Ultra Wide ( up to two controller)composed by:

- PM3334UW SCSI3 Raid UWIDE Adapter  GO2173  	cod. 212803U

- SX4030/1UW single SCSI module  	IF 2065  	cod. 212804V

- SX4030/2UW dual SCSI module 	IF 2066  	cod. 212805W

SM4000/4  SIMM memory module 	ME 2027 	cod. 562227Y

- ADPTEC 2940 UW   (SCSI3 UW controller )

## 4.9-SCSI configuration

- Default : removable and HDU connected to the SCSI embedded on MB

- Duplexing: removable and one set (up to two) of HDU connected to the SCSI embedded on MB, one set (up to two) connected to the Adaptec 2940UW  (plugged in slot 3)

- Raid : removable connected to the SCSI embedded on MB, HDU connected to the DPTUW board (plugged in slot 2). An additional DPT  UW board must be plugged on slot 3.

- PEM  (up two PEM): pheripheral expansion module can be connected only to the DPT UW board. The transfer rate must be set to 20Mbyte/sec max.

## 4.10-Tested Network Card

The table below shows the boards tested with the released OS.

| WNT 3.51 Server + SP #4 | OC-2185; OC-2315 |
| --- | --- |
| WNT 4.0 Server | 3C905 |
|  | Intel PRO100/B On Board Chip-Set |
| OS/2 WARP 3.0 | OC-3137 |
|  | ZX312 |
|  | Intel PRO100/B On Board Chip-Set |
| NetWare 4.11 | ZX312 |
| NetWare 3.12 | NE2000+ |
|  | 3C590 ; 3C900 |
|  | OC-3137 |
|  | Intel PRO100/B On Board Chip-Set |
| SCO 5.02 | OC-3137;OC3118;OC2325 |
|  | Z’nix ZX312 |
|  | 3C900 |
|  | Intel PRO100/B On Board Chip-Set |
| SCO UNIXWARE 2.1 | Intel PRO100/B On Board Chip-Set |

# 5-Warnings & Limitations

Z’nyx 312  should be used in PCI slots 1 or  2 with OS/2 warp 3.0.

In Novell Netware 3.12 and 4.11 DPT UW boards cannot be use the IRQ9 interrupt line (interrupt suggested=5).

The installation phase of SCO Open Server 5.02 needs the CD-rom and the boot HDU connected by the same SCSI channel in the case of homogeneous controller.

On RAID applications is mandatory to use the Uninterruptible Power  Supply  (UPS)

On applications where PEM is used is mandatory turn on/off the system and PEM by only UPS, and the push-button (on/off) must be locked by the key on “ON” position .

Although Pentium Pro supports 36 bit addressing, the 440FX chipset only supports 32 bit addresses.